SED Sensor

| Goal | Morphological measurements |
| --- | --- |
| Typical area | Salt marshes, mangroves, tidal flats, dunes, beaches |
| Typical time resolution | Days–months |
| Investment costs | < 1000 € |
| Operation Costs |  |
| Labor | low |
| Spatial Coverage | spot |
| Time Frequency | continuous |
| Detail level | High (2 mm vertical resolution) |

Method

When in use, a SED-sensor is inserted vertically into the seabed, leaving ca. half of the measuring section above the seabed. During measurement, a current runs through each light sensitive cell, which is proportional to the amount of light on each cell. This current is measured over a resistor to give a voltage output. In this way, above-ground and below-ground cells will give higher and lower output accordingly.

Materials

- Sed Sensor

Additional information